# Spatial Analysis in R for Forest Carbon Verification

A Practical Training in GIS

Murphy, S

2026-07-30

Table of contents

# Preface

This is a practical, comprehensive introduction to geospatial data science with R, built for scientists, analysts, and auditors who need to move confidently from a stack of shapefiles and satellite imagery to a defensible result. It teaches the foundations every analyst of spatial data must understand, how spatial data are represented, referenced, measured, combined, and monitored through time, using free and open tools, and it assumes some statistics but no prior GIS.

The course is organised as nine short chapters that run from simple to complex. Each teaches a foundational concept on one running dataset, then closes with a worked example drawn from a real forest-carbon verification project, so the concept earns its example and the example proves the concept. By the end you will have carried a single forest plot from raw coordinates through vector and raster analysis, terrain, LiDAR, disturbance detection, and uncertainty, and worked two capstones: a time-series analysis that ends in a quantified emission, and a spatial-pattern analysis of forest regeneration.

## The chapters

- Getting Started — reproducible R projects, the spatial stack, vector versus raster, a first map and coordinate check.

- Vector Data — geometries, attributes and support, spatial joins; closing on an area check.

- Raster Data — grids, map algebra, reclassification, extraction; closing on land-cover reclassification.

- Terrain and Hydrology — elevation, slope and aspect, flow; closing on inundation mapping.

- LiDAR and Point Clouds — returns, ground and canopy, tree detection; closing on a LiDAR forest inventory.

- Disturbance and Risk — change detection, accuracy assessment, risk modelling; closing on VM0048 risk and ACR accuracy.

- Uncertainty — sampling error, Monte Carlo propagation, conservative estimates; closing on area-adjusted accuracy.

- Time Series (capstone I) — space-time cubes, trends, change to emissions; closing on IPCC Tier 1 fire emissions.

- Spatial Patterns (capstone II) — point patterns, intensity, nearest-neighbour, Ripley’s K; closing on post-fire conifer regeneration.

An appendix covers the surrounding discipline: automation, spatial-data protocols, and registry practice.

## The running dataset

Every chapter draws on one dataset: a mapped forest-dynamics plot modelled on the Smithsonian ForestGEO census at the Conservation Biology Institute in Front Royal, Virginia. It records 2,287 stems in a 400 by 640 metre plot, each identified to species and measured for diameter. A forestry dataset is incidental to the methods, which are general to all spatial data, but it gives the examples a consistent grounding and lets the applied exercises connect to real problems in inventory, carbon, and monitoring.

About the data

The stem list (species and diameter) follows the published ForestGEO census used in the companion uncertainty book. The within-plot coordinates were generated across the real plot footprint because the source census table omitted them, so the maps are representative rather than a survey of record. Terrain, imagery, LiDAR, and time-series layers are simulated over the same footprint so the book renders offline and reproducibly, and every synthetic layer says so where it is built. The keystone closings describe workflows from real verification projects and reproduce their analytical core on this shared data. The one exception is the second capstone (Chapter 9), which runs on the real, published coordinates of a post-fire conifer regeneration survey in the Selkirk Mountains of British Columbia (Murphy et al., 2026).

## Using this book

Each chapter is a self-contained lesson that builds on the last. Code is shown in full and is meant to be run; the fastest way to learn is to work through it line by line and then attempt the exercises. Readers who want a deeper or more general treatment are well served by the canonical texts in the field (Bivand et al., 2013; Lovelace et al., 2019; Pebesma & Bivand, 2023), and the sf and terra packages that underpin the modern R-spatial stack are described by Pebesma (2018) and Hijmans (2024). Where the applied material touches forest-carbon conventions it follows IPCC (2006).

# 1. Getting Started

## 1.1 Reproducibility

An audit stands or falls on whether someone else can reproduce the number. A spreadsheet built with forty manual clicks cannot be re-run, cannot be reviewed line by line, and cannot be trusted a year later when the inputs have changed. A script can. This is the reason to do spatial work in R: every decision, from the projection you chose to the buffer distance you applied, is written down and repeatable.

R is a free, open-source language with a mature ecosystem for geographic data. In one session you can read a shapefile a surveyor sent you, pull an elevation model from an open server, overlay the two, extract terrain at your field plots, fit a model, and export a map, without leaving the environment or paying for a licence. For a forest-carbon or agriculture auditor that means the whole chain from raw data to reported estimate lives in one reviewable document. This course uses the modern R-spatial stack, built on two packages that have become the standard tools of the field (Hijmans, 2024; Pebesma, 2018).

## 1.2 The stack

You will meet these packages throughout the book. Install them once before you begin.

| Package | Role |
| --- | --- |
| sf | Vector data: points, lines, polygons |
| terra | Raster data: imagery, elevation, gridded surfaces |
| dplyr / tidyverse | Data wrangling and pipes |
| tmap | Thematic maps, static and interactive |
| ggplot2 | Statistical graphics |
| gstat | Geostatistics and interpolation |
| spdep | Spatial autocorrelation and neighbours |

To install everything the course needs, run this once in the console. It is set not to execute when the book renders.

install.packages(c(
  "sf", "terra", "dplyr", "tidyverse", "tmap",
  "ggplot2", "gstat", "spdep", "elevatr", "rnaturalearth"
))

Load the core three at the top of any spatial script:

library(sf)
library(terra)
library(dplyr)

The messages sf and terra print on loading name the versions of GDAL, GEOS and PROJ you are linked against. Note them: a projection or geometry result can occasionally differ between library versions.

## 1.3 Projects

Before loading data, put your work in an RStudio Project, a folder with an .Rproj marker. It fixes two things that otherwise cause endless confusion: it sets the working directory to the project root, and it lets you refer to data with short relative paths like data/scbi_stems.csv instead of a full path tied to your machine. A tidy layout looks like:

training-rspatial/
├── data/                 # input datasets
├── 01-getting-started.qmd
├── 02-vector-data.qmd
└── training-rspatial.Rproj

With a project open, every path in this book that begins with data/ resolves correctly.

## 1.4 The running dataset

One dataset runs through the whole course: a mapped forest-dynamics plot, modelled on the Smithsonian ForestGEO plot at the Conservation Biology Institute in Front Royal, Virginia. Every stem in a 400 by 640 metre plot is stem-mapped, identified to species, and measured for diameter. It is an ideal teaching set because it is large, real in structure, and every tree carries a location.

About the data

The stem list (species, diameter) follows the published ForestGEO census used in the companion uncertainty book. The within-plot coordinates were generated across the real plot footprint because the source census table omitted them, so the map is representative rather than a survey of record. Terrain, imagery and LiDAR layers in later chapters are simulated over the same footprint for a clean, always-runnable progression, and every synthetic layer says so where it is built.

The stem table is an ordinary CSV, so base R reads it without any spatial machinery:

stems <- read.csv("data/scbi_stems.csv")
str(stems)
#> 'data.frame':    2287 obs. of  10 variables:
#>  $ treeID : int  2695 1229 1230 1295 1229 66 2600 4936 1229 1005 ...
#>  $ stemID : int  2695 38557 1230 32303 32273 31258 2600 4936 36996 1005 ...
#>  $ dbh    : num  1.4 1.7 1.4 1 2.5 2.2 1.8 1.4 2 1.4 ...
#>  $ genus  : chr  "Acer" "Acer" "Acer" "Acer" ...
#>  $ species: chr  "negundo" "negundo" "negundo" "negundo" ...
#>  $ family : chr  "Sapindaceae" "Sapindaceae" "Sapindaceae" "Sapindaceae" ...
#>  $ gx     : num  188 122 164 106 325 ...
#>  $ gy     : num  466 568 459 157 319 ...
#>  $ lon    : num  -78.1 -78.1 -78.1 -78.1 -78.1 ...
#>  $ lat    : num  38.9 38.9 38.9 38.9 38.9 ...

Two functions tell you almost everything about a new dataset. summary() shows the distribution of each column; a grouped dplyr summary reads the forest:

summary(stems[, c("dbh", "gx", "gy")])
#>       dbh              gx              gy       
#>  Min.   : 1.00   Min.   :  0.2   Min.   :  0.0  
#>  1st Qu.: 1.20   1st Qu.: 99.1   1st Qu.:160.9  
#>  Median : 1.70   Median :202.4   Median :321.7  
#>  Mean   : 5.52   Mean   :201.5   Mean   :322.4  
#>  3rd Qu.: 3.40   3rd Qu.:302.0   3rd Qu.:481.4  
#>  Max.   :92.00   Max.   :400.0   Max.   :639.9

stems |>
  group_by(genus) |>
  summarise(n = n(), mean_dbh = round(mean(dbh), 1)) |>
  arrange(desc(n)) |>
  head(8)
#> # A tibble: 8 × 3
#>   genus           n mean_dbh
#>   <chr>       <int>    <dbl>
#> 1 Lindera      1201      1.6
#> 2 Rosa          151      1.5
#> 3 Carya         137     12.7
#> 4 Amelanchier   123      3.1
#> 5 Hamamelis     107      2.6
#> 6 Quercus        84     27.8
#> 7 Fraxinus       76     23.6
#> 8 Carpinus       57      5.4

Diameters run from saplings at 1 cm to canopy trees near a metre, and a handful of genera dominate the stem count. That is the size and composition structure every biomass and carbon calculation later in the book will rest on.

## 1.5 Vector and raster

Spatial data come in two great families, and knowing which you hold is the first decision in any analysis. Vector data represent discrete objects by their geometry: a tree is a point, a road a line, a stand boundary a polygon, each carrying attributes in a table. Raster data represent a continuous surface as a grid of equal cells, each holding one value: elevation, reflectance, canopy height. Our stems are vector; the elevation and imagery we bring in later are raster. Most real analysis moves between the two, and the middle chapters of this book are about doing that cleanly.

## 1.6 A first map

The CSV knows nothing about geography yet: lon and lat are just numbers. sf turns them into geometry by naming the coordinate columns and the coordinate reference system, here WGS 84 longitude/latitude, whose EPSG code is 4326:

stems_sf <- st_as_sf(stems, coords = c("lon", "lat"), crs = 4326)
stems_sf
#> Simple feature collection with 2287 features and 8 fields
#> Geometry type: POINT
#> Dimension:     XY
#> Bounding box:  xmin: -78.1454 ymin: 38.8935 xmax: -78.14078 ymax: 38.89925
#> Geodetic CRS:  WGS 84
#> First 10 features:
#>    treeID stemID dbh genus species      family    gx    gy
#> 1    2695   2695 1.4  Acer negundo Sapindaceae 188.0 466.1
#> 2    1229  38557 1.7  Acer negundo Sapindaceae 121.5 567.9
#> 3    1230   1230 1.4  Acer negundo Sapindaceae 164.0 458.6
#> 4    1295  32303 1.0  Acer negundo Sapindaceae 106.1 156.9
#> 5    1229  32273 2.5  Acer negundo Sapindaceae 325.0 318.9
#> 6      66  31258 2.2  Acer negundo Sapindaceae 166.3 465.8
#> 7    2600   2600 1.8  Acer negundo Sapindaceae 385.3 198.1
#> 8    4936   4936 1.4  Acer negundo Sapindaceae 281.6 332.4
#> 9    1229  36996 2.0  Acer negundo Sapindaceae 292.5 639.8
#> 10   1005   1005 1.4  Acer negundo Sapindaceae  82.6 481.7
#>                      geometry
#> 1  POINT (-78.14323 38.89769)
#> 2     POINT (-78.144 38.8986)
#> 3  POINT (-78.14351 38.89762)
#> 4  POINT (-78.14418 38.89491)
#> 5  POINT (-78.14165 38.89637)
#> 6  POINT (-78.14348 38.89768)
#> 7  POINT (-78.14095 38.89528)
#> 8  POINT (-78.14215 38.89649)
#> 9  POINT (-78.14202 38.89925)
#> 10 POINT (-78.14445 38.89783)

Notice what sf prints: the geometry type (POINT), the bounding box, the CRS (Geodetic CRS: WGS 84), and the attribute table with a geometry column on the end. An sf object is a data frame that also knows where each row sits on Earth. The plot layer also ships as a shapefile, read the same way with st_read("data/scbi_stems.shp").

Plotting one attribute draws a map coloured by that value. We map the larger stems to keep the picture legible:

canopy <- stems_sf[stems_sf$dbh >= 20, ]
plot(canopy["dbh"], pch = 19, cex = 0.5, main = "Stems ≥ 20 cm DBH")

There is the plot: canopy trees scattered across the footprint, shaded by diameter. From here the course is about doing this properly.

## 1.7 First checks

The first thing to do on any real project is not analysis but a coordinate sanity check. A layer whose CRS is wrong, or missing, will silently misplace every downstream result, and the error is invisible on a plot that looks fine on its own axes. Two questions answer it: does the layer declare a CRS, and does it land where it should on Earth?

st_crs(stems_sf)$epsg              # 4326: the layer knows its CRS
#> [1] 4326
st_bbox(stems_sf)                  # does the extent sit where you expect?
#>      xmin      ymin      xmax      ymax 
#> -78.14540  38.89350 -78.14078  38.89925

The bounding box should fall over Front Royal, Virginia (near 78.1° W, 38.9° N). If it landed in the Gulf of Guinea at (0, 0), the coordinates were never georeferenced. This is exactly the opening move of any mapping template: read the layer, confirm its CRS, confirm its extent, and only then begin. Every keystone project in this book starts here, because a defensible result begins with knowing your data are where they claim to be.

## 1.8 Exercises

- Read scbi_stems.csv and report the number of stems, the number of distinct genera, and the mean and maximum diameter. Which genus has the most stems?

- Convert the table to an sf object with st_as_sf(), then confirm its CRS with st_crs() and its extent with st_bbox(). In one sentence, why is checking the CRS the first thing you should do?

- Map the stems coloured by genus for the three most common genera only. Do they appear mixed or spatially segregated?

- Add a basal_area column (the cross-sectional area of each stem, pi * (dbh/2)^2 in cm²) and report total basal area by genus. Which genus dominates the stand by basal area rather than by stem count?

# 2. Vector Data

## 2.1 Geometry types

Vector data represent the world as discrete shapes. The simple-features standard, which sf implements, defines a small family of geometry types built from coordinates: a point is a single location, a linestring an ordered path, a polygon an enclosed area, and each has a multi form for features made of several parts. Almost everything an auditor handles is one of these: stems and plot centres are points, streams and roads are lines, stands and ownership parcels are polygons.

library(sf)
library(dplyr)

stems <- st_as_sf(read.csv("data/scbi_stems.csv"),
                  coords = c("lon", "lat"), crs = 4326)
st_geometry_type(stems)[1]
#> [1] POINT
#> 18 Levels: GEOMETRY POINT LINESTRING POLYGON MULTIPOINT ... TRIANGLE

Every metric operation, area, distance, overlap, is meaningless on longitude and latitude, because a degree is not a fixed length. So the first move in vector analysis is to project to a coordinate system whose units are metres. For this plot in Virginia that is UTM zone 17 North, EPSG 32617:

stems <- st_transform(stems, 32617)
st_bbox(stems)          # now in metres
#>      xmin      ymin      xmax      ymax 
#>  747559.3 4308835.3  747976.6 4309480.1

## 2.2 sf anatomy

An sf object is a table whose rows are features and whose last column, of class sfc, holds the geometry. Because it is a data frame first and spatial second, the whole dplyr grammar applies and the geometry is carried along automatically:

stems |>
  filter(dbh >= 30) |>
  mutate(basal_area_cm2 = pi * (dbh / 2)^2) |>
  select(treeID, genus, dbh, basal_area_cm2) |>
  head()
#> Simple feature collection with 6 features and 4 fields
#> Geometry type: POINT
#> Dimension:     XY
#> Bounding box:  xmin: 747567.3 ymin: 4308977 xmax: 747908.4 ymax: 4309385
#> Projected CRS: WGS 84 / UTM zone 17N
#>   treeID genus  dbh basal_area_cm2                 geometry
#> 1   1139  Acer 44.0      1520.5308 POINT (747859.4 4309329)
#> 2   2436  Acer 37.4      1098.5835 POINT (747648.9 4309107)
#> 3   1134  Acer 60.0      2827.4334 POINT (747588.6 4308977)
#> 4   1031  Acer 42.5      1418.6254 POINT (747567.3 4309385)
#> 5   3800  Acer 33.2       865.6973 POINT (747908.4 4309047)
#> 6     29  Acer 32.6       834.6898 POINT (747905.6 4309230)

st_drop_geometry() turns the layer back into an ordinary table when you want a plain summary, and st_geometry() extracts just the shapes when you want to plot outlines with no attribute shading.

## 2.3 Building polygons

Analysis usually needs areas, not just points. We divide the plot into 100-metre quadrats with st_make_grid(), the standard way to impose a regular sampling frame, and keep only the cells that cover the stems:

grid <- st_make_grid(stems, cellsize = 100) |> st_as_sf()
grid$quadrat <- seq_len(nrow(grid))
grid <- grid[lengths(st_intersects(grid, stems)) > 0, ]
nrow(grid)
#> [1] 34

## 2.4 Spatial joins

A spatial join attaches attributes by location rather than by a shared key. st_join() labels each stem with the quadrat that contains it, after which the counting is ordinary dplyr:

stems_q <- st_join(stems, grid, join = st_within)

per_quadrat <- stems_q |>
  st_drop_geometry() |>
  group_by(quadrat) |>
  summarise(n_stems = n(),
            mean_dbh = round(mean(dbh), 1))
head(per_quadrat)
#> # A tibble: 6 × 3
#>   quadrat n_stems mean_dbh
#>     <int>   <int>    <dbl>
#> 1       1      80      5.8
#> 2       2     100      4.8
#> 3       3      98      4  
#> 4       4      76      6.6
#> 5       5       9      2  
#> 6       6      62      7.4

This point-in-polygon join is the workhorse of inventory: it is how you assign field stems to strata, plots to ownership parcels, or detections to management units.

## 2.5 Support

An attribute value stands in a particular relationship to the geometry it is attached to, and getting that relationship wrong is one of the most common and least visible errors in spatial analysis. The support of an attribute is the area over which it was measured or to which it applies. A stem count has each quadrat as its support; a per-hectare density refers to a hectare. Two numbers can look identical yet mean different things because their support differs.

Attributes divide into two kinds by how they behave when a geometry changes size. An extensive attribute is a total that scales with area, a count of stems or a tonnage of carbon; split a polygon and the value divides between the pieces. An intensive attribute is a rate or density that does not scale, stems per hectare or mean diameter; split the polygon and each piece keeps the value. The distinction dictates how you aggregate: sum the extensive total, average the intensive rate.

grid_join <- left_join(grid, per_quadrat, by = "quadrat")
grid_join$area_ha  <- as.numeric(st_area(grid_join)) / 1e4     # support, in ha
grid_join$stems_ha <- grid_join$n_stems / grid_join$area_ha    # intensive

# extensive -> sum; intensive -> area-weighted mean
c(total_stems = sum(grid_join$n_stems),
  mean_density = round(weighted.mean(grid_join$stems_ha, grid_join$area_ha), 0))
#>  total_stems mean_density 
#>         2285           67

Summing a density, or averaging a total, is simply wrong, and it is exactly how reported area totals silently inflate. Always be able to state the support of a number, and never combine values of different support without an explicit rule.

plot(grid_join["stems_ha"], main = "Stem density (stems/ha)")

## 2.6 Keystone: area checks

The applied heart of vector GIS in verification work is the area check: confirming that the area a project claims matches the area its geometry actually encloses, and that its parcels do not overlap or leave gaps. Overstated area is overstated crediting, so this is often the first quantitative finding in an audit. The tools are exactly those above, st_area, st_is_valid, st_intersection, applied to the project’s own polygons.

We stand in for the project boundary with the plot’s nominal extent and check it against the mapped stems. First, geometry validity, because area and overlap operations on an invalid polygon return nonsense:

boundary <- st_as_sfc(st_bbox(stems)) |> st_sf(id = "claimed_boundary")
all(st_is_valid(boundary))
#> [1] TRUE

Now the area reconciliation. A claimed area is compared with the geometry’s measured area; the auditor reports the difference:

claimed_ha  <- 25.6                                   # nominal 400 x 640 m plot
measured_ha <- as.numeric(st_area(boundary)) / 1e4

data.frame(
  claimed_ha  = claimed_ha,
  measured_ha = round(measured_ha, 2),
  difference_ha = round(measured_ha - claimed_ha, 2),
  pct_error   = round(100 * (measured_ha - claimed_ha) / claimed_ha, 1)
)
#>   claimed_ha measured_ha difference_ha pct_error
#> 1       25.6        26.9           1.3       5.1

Finally, overlap detection between parcels. Where two subareas are submitted, their intersection must be empty; any positive intersection area is double-counted land:

a <- st_as_sfc(st_bbox(c(xmin = 747000, xmax = 747250,
                         ymin = 4306000, ymax = 4306400), crs = 32617))
b <- st_as_sfc(st_bbox(c(xmin = 747200, xmax = 747450,
                         ymin = 4306000, ymax = 4306400), crs = 32617))
overlap <- st_intersection(a, b)
as.numeric(st_area(overlap)) / 1e4          # hectares of double-counted area
#> [1] 2

A non-zero result is a finding: those hectares are claimed twice and must be reconciled before any area total is accepted. The full project workflow layers on shapefile ingest, coordinate-system checks, and a written report, but the logic is this: validate the geometry, measure the area, compare to the claim, and test every parcel pair for overlap.

## 2.7 Exercises

- Project the stems to UTM 17N and build a 50-metre quadrat grid. How many quadrats cover the plot, and how does the mean stem count per quadrat change from the 100-metre version? Relate this to support.

- Classify n_stems and stems_ha as extensive or intensive, and state which should be summed and which averaged when quadrats are merged into blocks.

- Take the claimed_ha of 25.6 and imagine the true mapped boundary were 5% smaller. Compute the over-crediting in hectares and explain why an auditor treats positive area error differently from negative.

- (Applied.) Two submitted parcels overlap by 3 ha out of a claimed 40 ha total. Write the st_intersection and st_area calls that quantify the overlap, and state the corrected non-overlapping total.

# 3. Raster Data

## 3.1 The raster model

A raster represents a surface as a grid of equal cells, each holding one value. Where vector data draw discrete objects, rasters describe things that vary continuously across space, elevation, reflectance, temperature, canopy height, and the things we derive from imagery: land cover, disturbance, biomass. The data model is nothing more than a matrix plus the georeferencing that ties cell (1,1) to a place and fixes the cell size. terra is the modern package for it (Hijmans, 2024).

We build a raster over the study plot by summarising the stems onto a grid. First project the stems and give each a basal area, then lay down a 10-metre grid and total the basal area falling in each cell:

library(terra)
library(sf)

stems_sf <- st_as_sf(read.csv("data/scbi_stems.csv"),
                     coords = c("lon", "lat"), crs = 4326) |>
  st_transform(32617)
stems_sf$ba <- pi * (stems_sf$dbh / 2)^2 / 1e4     # basal area, m² per stem

stems <- vect(stems_sf)
r  <- rast(ext(stems), resolution = 10, crs = "EPSG:32617")
ba <- rasterize(stems, r, field = "ba", fun = sum, background = 0)
names(ba) <- "basal_area"
ba
#> class       : SpatRaster 
#> size        : 64, 42, 1  (nrow, ncol, nlyr)
#> resolution  : 9.934985, 10.07471  (x, y)
#> extent      : 747559.3, 747976.6, 4308835, 4309480  (xmin, xmax, ymin, ymax)
#> coord. ref. : WGS 84 / UTM zone 17N (EPSG:32617) 
#> source(s)   : memory
#> name        : basal_area 
#> min value   :  0.0000000 
#> max value   :  0.6649935

The printout is the raster data model made concrete: rows and columns, cell size, extent, CRS, and the value range. Each 10-metre cell now holds the total stem basal area within it, a crude but genuine biomass proxy.

## 3.2 Inspecting rasters

The same handful of accessors describe any raster:

res(ba)                                        # cell size, metres
#> [1]  9.934985 10.074715
ext(ba)                                        # bounding box
#> SpatExtent : 747559.294472252, 747976.563837432, 4308835.31323516, 4309480.09499186 (xmin, xmax, ymin, ymax)
global(ba, c("min", "max", "mean"))            # value range
#>            min       max        mean
#> basal_area   0 0.6649935 0.009831975
plot(ba, main = "Basal area per 10 m cell (m²)")

## 3.3 Raster algebra

Because a raster is a grid of numbers, arithmetic on rasters is arithmetic on cells. This is map algebra, and it is why continuous surfaces are so easy to work with. Converting basal area to an aboveground-carbon proxy is one expression applied to every cell at once:

carbon <- ba * 12          # illustrative MgC per cell from basal area
names(carbon) <- "carbon"

Logical operators work the same way and return a mask of ones and zeros. A “stocked” mask, cells carrying any measurable basal area, is a single comparison:

stocked <- ba > 0.05
plot(stocked, main = "Stocked cells")

## 3.4 Reclassification

Turning a continuous surface into a small number of classes is reclassification, the operation behind every land-cover and zonation map. classify() takes a matrix of from–to–becomes rows and cuts the surface at the thresholds:

rcl <- matrix(c(-Inf, 0.05, 1,     # sparse
                0.05, 0.30, 2,     # moderate
                0.30, Inf, 3),     # dense
              ncol = 3, byrow = TRUE)
zones <- classify(ba, rcl)
levels(zones) <- data.frame(id = 1:3, class = c("sparse", "moderate", "dense"))
plot(zones, main = "Stocking classes")

## 3.5 Extraction

The bridge from raster back to field data is extraction: reading the raster value at each point location. terra::extract() samples the surface wherever a plot, a tree, or a sample sits, which is how gridded covariates enter a plot-level model:

pts <- spatSample(as.polygons(ext(ba), crs = crs(ba)), 5, method = "random")
extract(ba, pts)
#>   ID   basal_area
#> 1  1 4.908739e-04
#> 2  2 1.327323e-04
#> 3  3 0.000000e+00
#> 4  4 7.853982e-05
#> 5  5 0.000000e+00

Extraction and reclassification are the two operations that connect the raster and vector worlds, and everything applied in the rest of the book uses one or both.

## 3.6 Keystone: land-cover reclassification

Remote-sensing verification turns imagery into a land-use / land-cover (LULC) map and then reasons about it: how much forest, how much converted, how the classes change through time. The Bolivia national land-cover work does this at country scale; the mechanics are the reclassification and area accounting shown here, which we later carry into the uncertainty chapter to attach a confidence interval to each class area.

We stand in for a spectral index with a smooth synthetic surface over the plot, classify it into cover types by threshold, then reclassify to the forest / non-forest split that most carbon accounting turns on:

set.seed(1)
xy  <- xyFromCell(r, 1:ncell(r))
val <- 0.55 + 0.30 * sin(xy[, 1] / 120) + 0.20 * cos(xy[, 2] / 160) +
       rnorm(ncell(r), 0, 0.05)
ndvi <- setValues(r, pmin(pmax(val, -0.1, na.rm = TRUE), 0.95))
names(ndvi) <- "ndvi"

# thresholds -> cover classes
lulc <- classify(ndvi, matrix(c(-Inf, 0.2, 1,     # water/bare
                                 0.2, 0.5, 2,      # grass/shrub
                                 0.5, Inf, 3),     # forest
                              ncol = 3, byrow = TRUE))
levels(lulc) <- data.frame(id = 1:3,
                           cover = c("water/bare", "grass/shrub", "forest"))
plot(lulc, main = "Land cover from a spectral index")

The class areas are the deliverable. freq() counts cells per class, and multiplying by the cell area gives hectares:

cell_ha <- prod(res(lulc)) / 1e4
f <- freq(lulc)
f$area_ha <- round(f$count * cell_ha, 2)
f[, c("value", "count", "area_ha")]
#>         value count area_ha
#> 1  water/bare   828    8.29
#> 2 grass/shrub  1500   15.01
#> 3      forest   360    3.60

Reclassifying the three classes into a forest mask, the exact step behind a forest-area or deforestation figure, is one more classify():

forest <- classify(lulc, matrix(c(1, 0, 2, 0, 3, 1), ncol = 2, byrow = TRUE))
forest_ha <- global(forest, "sum", na.rm = TRUE)[1, 1] * cell_ha
round(forest_ha, 1)                             # hectares of forest
#> [1] 3.6

That single number, forest area, is what a project claims and an auditor recomputes. Chapter 7 returns to it to ask how certain the number is, because a class area is only as good as the map’s accuracy.

## 3.7 Exercises

- Build the basal-area raster at 20-metre resolution instead of 10. How do the mean cell value and the number of cells change, and why does total basal area stay the same?

- Reclassify the basal-area surface into four stocking classes and report the area in hectares of each. Which class covers the most ground?

- Extract the basal-area value at the location of every stem with extract(). Why is the extracted value not the same as the stem’s own basal area?

- (Applied.) Change the forest threshold on the synthetic index from 0.5 to 0.45 and recompute forest area. By how many hectares does the estimate move, and what does that sensitivity imply for defining a class boundary in a real LULC map?

# 4. Terrain and Hydrology

## 4.1 Elevation

Terrain is the covariate behind almost everything else in a forest: it drives productivity, access, erosion, and where water collects. It arrives as a digital elevation model (DEM), a raster whose cell value is height above a datum. In operational work you download one, and the elevatr package fetches open tiles for any area:

library(elevatr)
aoi <- st_as_sf(data.frame(x = -78.143, y = 38.896),
                coords = c("x", "y"), crs = 4326)
dem <- get_elev_raster(aoi, z = 13) |> rast()

So the chapter runs offline and reproducibly, we build a DEM with a valley over the study plot instead. The surface tilts gently and is cut by a central channel, the kind of relief that governs drainage:

library(terra)
library(sf)

stems <- st_as_sf(read.csv("data/scbi_stems.csv"),
                  coords = c("lon", "lat"), crs = 4326) |>
  st_transform(32617)

dem <- rast(ext(vect(stems)), resolution = 10, crs = "EPSG:32617")
xy  <- xyFromCell(dem, 1:ncell(dem))
ymid <- mean(xy[, 2])
elev <- 120 + 0.03 * (xy[, 1] - min(xy[, 1])) -
        18 * exp(-((xy[, 2] - ymid) / 70)^2)
dem  <- setValues(dem, elev)
names(dem) <- "elevation"
global(dem, c("min", "max", "mean"))
#>               min    max     mean
#> elevation 102.093 132.22 122.6464

## 4.2 Terrain derivatives

Elevation is rarely the variable of interest; what it implies, slope, aspect, ruggedness, is. terrain() derives these directly. Slope and aspect need a projected DEM so horizontal and vertical distances share units, which ours already is:

slope  <- terrain(dem, "slope",  unit = "degrees")
aspect <- terrain(dem, "aspect", unit = "degrees")
tri    <- terrain(dem, "TRI")            # terrain ruggedness index
plot(c(slope, tri), main = c("Slope (deg)", "Ruggedness (TRI)"))

Slope matters for operability and erosion risk; ruggedness is a useful productivity covariate because sheltered, gentler terrain often carries taller, denser stands. A hillshade turns terrain into an intuitive relief image by simulating illumination, the standard base layer under any map:

hs <- shade(terrain(dem, "slope",  unit = "radians"),
            terrain(dem, "aspect", unit = "radians"),
            angle = 40, direction = 315)
plot(hs, col = grey(0:100 / 100), legend = FALSE, main = "Hillshade")

## 4.3 Flow direction

Water moves downhill, so a DEM already encodes drainage. terrain() computes flow direction, the neighbour each cell drains to, which is the first step of every hydrological routing algorithm and the input to catchment delineation:

flowdir <- terrain(dem, "flowdir")
plot(flowdir, main = "Flow direction")

Full watershed delineation, flow accumulation, stream networks, catchment boundaries, is the domain of dedicated tools such as whitebox, which route flow across the accumulated grid. The principle is the one shown here: every hydrological product begins with elevation and the slope and flow fields derived from it.

## 4.4 Keystone: hydrological analysis

Wetland and inundation mapping asks a direct question of a DEM: given a water level, what land floods? The Lake Chilwa inundation work answers it across a shallow basin whose area swings enormously with small changes in level. The method is a threshold on elevation, and it is the same logic whether the water is a lake, a floodplain, or a reservoir behind a proposed dam.

We treat the valley floor as the basin. Flooded cells are those below a chosen water level, and the flooded area is the deliverable:

water_level <- min(values(dem), na.rm = TRUE) + 3   # 3 m above the channel floor
flooded <- dem < water_level
cell_ha <- prod(res(dem)) / 1e4
flooded_ha <- global(flooded, "sum", na.rm = TRUE)[1, 1] * cell_ha
round(flooded_ha, 2)                                  # hectares inundated
#> [1] 0.46

plot(hs, col = grey(0:100 / 100), legend = FALSE, main = "Inundation at +3 m")
plot(flooded, add = TRUE, alpha = 0.5, col = c(NA, "steelblue"), legend = FALSE)

The value of the method is the rating curve: sweep the water level and record the area at each, and you have the relationship between stage and flooded extent that drives flood risk, wetland carbon, and reservoir planning:

levels <- seq(min(values(dem), na.rm = TRUE) + 1,
              min(values(dem), na.rm = TRUE) + 8, by = 1)
area_at <- sapply(levels, function(L)
  global(dem < L, "sum", na.rm = TRUE)[1, 1] * cell_ha)
data.frame(water_level_m = round(levels, 1), flooded_ha = round(area_at, 1))
#>   water_level_m flooded_ha
#> 1         103.1        0.1
#> 2         104.1        0.2
#> 3         105.1        0.5
#> 4         106.1        0.7
#> 5         107.1        0.9
#> 6         108.1        1.2
#> 7         109.1        1.6
#> 8         110.1        1.9

Reading down the table, a few metres of stage change a small flooded area into a large one, the non-linearity that makes shallow basins so sensitive and makes stating the assumed water level essential to any wetland-area claim. The real project adds a satellite-derived water mask to validate the DEM prediction, but the analytical core is this thresholded elevation surface.

## 4.5 Exercises

- Build the DEM and report its relief (max minus min elevation). Derive slope and describe where the steepest ground lies relative to the central valley.

- Compute a hillshade with the sun in the northeast (direction = 45) and compare it with the northwest illumination used above. Why does illumination direction change which slopes look bright?

- Recompute flooded area at water levels of +2, +4 and +6 m. Plot area against level: is the relationship linear? What does the shape tell you about the basin?

- (Applied.) A project claims a 50-hectare seasonal wetland. Explain how you would use a DEM, a water-level assumption, and the rating-curve method above to test that claim, and what single assumption most affects your answer.

# 5. LiDAR and Point Clouds

## 5.1 Point clouds

Airborne LiDAR measures the forest in three dimensions. A scanner fires laser pulses and times their return, and each pulse can produce several returns as it passes through the canopy: the first off the treetops, later ones off branches and understory, the last off the ground. The result is a dense point cloud, millions of x, y, z points, from which we reconstruct both the ground surface and the vegetation above it. Full-waveform sensors record the entire returned energy profile rather than discrete points, giving even finer vertical structure. The dedicated R package is lidR, which reads the LAS/LAZ files LiDAR ships in.

We illustrate the vertical structure with a simulated transect: a sloping ground surface with canopy returns scattered above it, exactly the mix a single scan line produces.

set.seed(1)
n <- 2000
x <- runif(n, 0, 100)
ground <- 100 + 0.05 * x
canopy <- runif(n) < 0.6
z <- ground + ifelse(canopy, rgamma(n, 2, 0.25), 0)

plot(x, z, pch = 20, cex = 0.4,
     col = ifelse(canopy, "forestgreen", "tan4"),
     xlab = "Distance (m)", ylab = "Elevation (m)",
     main = "LiDAR returns: ground (brown) and canopy (green)")

## 5.2 Ground and canopy

Two surfaces come out of a point cloud. Classifying the lowest returns as ground and interpolating them gives the digital terrain model (DTM), the bare earth. Taking the highest returns gives the digital surface model (DSM), the top of the canopy. Their difference is the quantity forestry cares about, the canopy height model (CHM):

CHM=DSM−DTM

Ground classification is the delicate step, separating last returns that hit soil from those that hit low vegetation, and lidR implements the standard algorithms (cloth simulation, progressive morphological filters). The principle is that height above ground, not height above sea level, is what relates to biomass.

## 5.3 Building a CHM

Rather than process raw LAS here, we construct a CHM from the stem map, since each tree’s height and crown define the canopy top. We give every stem a height from its diameter with a standard saturating growth curve, then rasterise the maximum height in each 5-metre cell:

library(terra)
library(sf)

stems <- st_as_sf(read.csv("data/scbi_stems.csv"),
                  coords = c("lon", "lat"), crs = 4326) |>
  st_transform(32617)
stems$height <- 1.3 + 27 * (1 - exp(-0.035 * stems$dbh))   # DBH → height (m)

v   <- vect(stems)
r   <- rast(ext(v), resolution = 5, crs = "EPSG:32617")
chm <- rasterize(v, r, field = "height", fun = max, background = 0)
names(chm) <- "canopy_height"
plot(chm, main = "Canopy height model (m)")

## 5.4 Tree detection

A CHM lets you find individual trees: each treetop is a local maximum, a cell higher than its neighbours. This is the basis of individual-tree detection, and the crown-radius-varying window in ForestTools refines it. We do the core operation directly, flagging cells that equal the maximum in a moving window and clear a minimum height:

win  <- focal(chm, w = matrix(1, 5, 5), fun = max, na.policy = "omit")
tops <- (chm == win) & (chm > 5)
tops[tops == 0] <- NA
top_pts <- as.points(tops)
nrow(top_pts)                              # detected treetops
#> [1] 269

plot(chm, main = "Detected treetops")
plot(top_pts, add = TRUE, pch = 3, cex = 0.6)

## 5.5 Keystone: lidar-forestry

The lidar-forestry workflow chains exactly these steps into an inventory: classify ground, build the DTM and CHM, detect treetops, segment crowns, and derive canopy metrics that predict biomass. The reason it matters for verification is that a CHM covers every square metre, where field plots sample a fraction of a percent, so LiDAR turns a handful of plots into a wall-to-wall map, provided the plot-to-LiDAR relationship is calibrated and its error carried through.

The bridge to biomass is a canopy metric summarised per unit area and regressed on plot biomass. Mean canopy height is the simplest such metric:

cell_ha <- prod(res(chm)) / 1e4
metrics <- data.frame(
  mean_canopy_height = round(global(chm, "mean", na.rm = TRUE)[1, 1], 1),
  p95_height         = round(quantile(values(chm), 0.95, na.rm = TRUE), 1),
  detected_trees     = nrow(top_pts),
  area_ha            = round(ncell(chm) * cell_ha, 1))
metrics
#>     mean_canopy_height p95_height detected_trees area_ha
#> 95%                  1        4.5            269    26.9

Detected trees will fall short of the true stem count, because the understory hides beneath the canopy and only dominant trees form local maxima. That gap is the central caution of LiDAR inventory: it sees the canopy, not the stand, and the correction comes from calibrating against field plots that do count every stem. The height metrics, by contrast, are robust and are what a LiDAR biomass model is built on. The full project fits that model and maps predicted biomass with its prediction error, the subject of the uncertainty chapter.

## 5.6 Exercises

- Rebuild the CHM at 2-metre resolution and describe how the number of detected treetops changes. Why does a finer grid tend to split single crowns into several maxima?

- Raise the minimum-height threshold in the detection step from 5 m to 15 m. How does the detected count change, and which trees are you now excluding?

- Compute the 25th, 50th, 75th and 95th height percentiles of the CHM. Explain why upper percentiles are better biomass predictors than the mean.

- (Applied.) You have LiDAR over 10,000 ha and field plots on 30 ha. Describe, in steps, how you would use the plots to calibrate a CHM-based biomass model and produce a wall-to-wall estimate, and where the two main sources of error enter.

# 6. Disturbance and Risk

## 6.1 Change detection

The core remote-sensing task in verification is finding where the forest changed. The simplest and most transparent method is image differencing: take a spectral index at two dates and subtract. Where vegetation was lost, the index drops; a threshold on the difference marks candidate disturbance. We build two dates of a synthetic index over the plot, the second with several patches cleared, and difference them.

library(terra)
library(sf)

stems <- st_as_sf(read.csv("data/scbi_stems.csv"),
                  coords = c("lon", "lat"), crs = 4326) |>
  st_transform(32617)
r <- rast(ext(vect(stems)), resolution = 10, crs = "EPSG:32617")

set.seed(7)
xy  <- xyFromCell(r, 1:ncell(r))
t1  <- setValues(r, 0.7 + 0.1 * sin(xy[, 1] / 100) + rnorm(ncell(r), 0, 0.03))

# time 2: same forest, minus three cleared blocks
t2 <- t1
clear <- (xy[, 1] > 747050 & xy[, 1] < 747130 & xy[, 2] > 4306100 & xy[, 2] < 4306180) |
         (xy[, 1] > 747250 & xy[, 1] < 747300 & xy[, 2] > 4306300 & xy[, 2] < 4306380)
t2[clear] <- 0.15 + rnorm(sum(clear), 0, 0.03)
names(t1) <- "ndvi_t1"; names(t2) <- "ndvi_t2"

change <- t2 - t1
plot(change, main = "NDVI change (t2 − t1)")

A negative threshold isolates loss. The disturbed area is the immediate deliverable, and the same number that drives an emissions estimate:

disturbed <- change < -0.3
cell_ha   <- prod(res(r)) / 1e4
dist_ha   <- global(disturbed, "sum", na.rm = TRUE)[1, 1] * cell_ha
round(dist_ha, 2)                                  # hectares of detected loss
#> [1] 0

## 6.2 Accuracy assessment

A map is a claim, and an unvalidated map is an unsupported claim. Accuracy assessment compares the map against reference data at a sample of points and summarises the agreement in a confusion matrix. From it come overall accuracy, and the per-class user’s and producer’s accuracies that ACR and VM0048 require, because a class area is only credible with its accuracy attached. We assess the disturbed / stable map against a reference sample:

set.seed(11)
ref <- data.frame(
  reference = sample(c("disturbed", "stable"), 120, replace = TRUE, prob = c(.3, .7)))
# a map that is mostly right, with some confusion
ref$mapped <- ifelse(ref$reference == "disturbed",
                     sample(c("disturbed", "stable"), 120, TRUE, c(.85, .15)),
                     sample(c("disturbed", "stable"), 120, TRUE, c(.10, .90)))

cm <- table(mapped = ref$mapped, reference = ref$reference)
cm
#>            reference
#> mapped      disturbed stable
#>   disturbed        20      7
#>   stable            2     91

overall <- sum(diag(cm)) / sum(cm)
users   <- diag(cm) / rowSums(cm)      # commission: of mapped X, how many correct
producers <- diag(cm) / colSums(cm)    # omission: of true X, how many caught
round(c(overall = overall), 3)
#> overall 
#>   0.925
round(rbind(users, producers), 3)
#>           disturbed stable
#> users         0.741  0.978
#> producers     0.909  0.929

Overall accuracy is the headline; the per-class figures are where problems hide. A high user’s but low producer’s accuracy for “disturbed” means the map misses real clearing, an omission error that understates loss, which is exactly the direction an auditor scrutinises. These same counts feed the area-adjusted estimator and its confidence interval in the next chapter.

## 6.3 Spatial risk

Some verification, additionality and permanence, turns on where disturbance is likely, not just where it happened. Risk modelling predicts that probability from spatial covariates. A robust and widely used predictor is proximity: disturbance begets disturbance, so distance to existing clearing is a strong risk covariate, the assumption behind deforestation-risk maps such as those in VM0048 baseline work. We compute distance to the detected disturbance and turn it into a risk surface:

disturbed[disturbed == 0] <- NA
dist_to_loss <- distance(disturbed)                 # metres to nearest clearing
risk <- exp(-dist_to_loss / 80)                     # decays with distance
names(risk) <- "risk"
plot(risk, main = "Relative deforestation risk")

Classifying the surface into zones gives the map a planner or auditor acts on, and the area in each zone:

risk_zone <- classify(risk, matrix(c(-Inf, 0.2, 1,
                                      0.2, 0.5, 2,
                                      0.5, Inf, 3), ncol = 3, byrow = TRUE))
levels(risk_zone) <- data.frame(id = 1:3, zone = c("low", "medium", "high"))
f <- freq(risk_zone); f$area_ha <- round(f$count * cell_ha, 1)
f[, c("value", "area_ha")]
#> [1] value   area_ha
#> <0 rows> (or 0-length row.names)

## 6.4 Keystone: disturbance checks and VM0048 risk

Put together, the chapter is the disturbance-check pipeline: difference two dates to detect loss, quantify the area, validate it with an accuracy assessment, and model where further loss is likely. VM0048 and the associated risk tools formalise the last step into a baseline of expected deforestation; the ACR accuracy requirements formalise the middle step into a mandatory confusion matrix. The auditor’s job is to reproduce each: recompute the change area, rebuild the confusion matrix from the reference sample, and check that the risk model’s covariates and thresholds are defensible. Every number here, hectares lost, overall accuracy, area at high risk, is one a project reports and a verifier must be able to regenerate.

## 6.5 Exercises

- Change the detection threshold from −0.3 to −0.2 and recompute disturbed area. Which way does the estimate move, and how does that trade omission against commission error?

- From the confusion matrix, compute the omission and commission error for the “disturbed” class. Which type of error causes a project to understate emissions, and why does that concern a verifier most?

- Rebuild the risk surface with a slower distance decay (divide by 160 instead of 80). How does the high-risk area change, and what real assumption does the decay rate encode?

- (Applied.) A project reports 200 ha of avoided deforestation with an overall map accuracy of 78%. Explain why the accuracy figure alone is not enough, and which per-class accuracies you would demand before accepting the area.

# 7. Uncertainty

## 7.1 Why uncertainty

Every estimate in this book is wrong by some amount, and the discipline of verification is knowing by how much. A carbon figure without an uncertainty is not conservative, it is simply incomplete, and the crediting standards, ART-TREES, VCS, ACR, all deduct credits in proportion to how uncertain the estimate is. Reducing uncertainty is therefore worth real money, and quantifying it honestly is the auditor’s core duty. This chapter builds the estimate, then its error, on the running stem data.

## 7.2 A carbon estimate

We convert diameters to aboveground biomass with a published generic hardwood equation (Jenkins et al., 2003), apply the IPCC default carbon fraction of 0.47 (IPCC, 2006), and scale to the plot area:

library(dplyr)
stems <- read.csv("data/scbi_stems.csv")

stems <- stems |>
  mutate(agb_kg   = exp(-2.48 + 2.4835 * log(dbh)),   # Jenkins generic hardwood
         carbon_Mg = 0.47 * agb_kg / 1000)

area_ha  <- 25.6
total_C  <- sum(stems$carbon_Mg)
C_per_ha <- total_C / area_ha
round(c(total_MgC = total_C, MgC_per_ha = C_per_ha), 1)
#>  total_MgC MgC_per_ha 
#>       82.3        3.2

That point estimate is the easy part. Three distinct sources of error sit beneath it: sampling error from measuring plots rather than the whole population, model error in the allometric equation, and parameter error in constants like the carbon fraction.

## 7.3 Sampling error

Divide the plot into quadrats and each becomes a sample of carbon density. The variation among them is the sampling uncertainty of the plot mean, summarised by the standard error and a confidence interval:

stems$quadrat <- paste(cut(stems$gx, 8), cut(stems$gy, 12))
quadrat_area  <- 25.6 / (8 * 12)                 # ha per quadrat (equal-area cut)

qd <- stems |>
  group_by(quadrat) |>
  summarise(carbon_Mg = sum(carbon_Mg), .groups = "drop") |>
  mutate(carbon_ha = carbon_Mg / quadrat_area)

m  <- mean(qd$carbon_ha); se <- sd(qd$carbon_ha) / sqrt(nrow(qd))
ci <- m + c(-1.96, 1.96) * se
round(c(mean = m, se = se, lower = ci[1], upper = ci[2]), 1)
#>  mean    se lower upper 
#>   3.2   0.4   2.5   3.9

The half-width relative to the mean is the number the standards care about: the wider it is, the deeper the deduction.

## 7.4 Monte Carlo propagation

Sampling error is only part of the story; the allometric model and the carbon fraction are uncertain too, and their errors compound. When error propagates through a non-linear function like a Db, the clean way to combine sources is Monte Carlo simulation: draw the uncertain inputs from their distributions many times, recompute the whole estimate each time, and read the spread of results (IPCC, 2006).

set.seed(42)
n_sim <- 2000
sim <- numeric(n_sim)
for (i in seq_len(n_sim)) {
  b  <- rnorm(1, 2.4835, 0.05)            # allometric exponent uncertainty
  cf <- rnorm(1, 0.47, 0.02)              # carbon-fraction uncertainty
  agb <- exp(-2.48 + b * log(stems$dbh))
  sim[i] <- sum(cf * agb / 1000) / area_ha
}
mc_ci <- quantile(sim, c(0.025, 0.975))
round(c(mean = mean(sim), lower = mc_ci[1], upper = mc_ci[2]), 1)
#>        mean  lower.2.5% upper.97.5% 
#>         3.3         2.2         4.7

hist(sim, breaks = 40, col = "grey80", border = "white",
     main = "Monte Carlo carbon density", xlab = "MgC / ha")
abline(v = mc_ci, col = "firebrick", lwd = 2, lty = 2)

## 7.5 Conservative estimates

The standards translate uncertainty into a deduction. A common rule is that when the half-width of the 90% or 95% interval exceeds a threshold (often 15% of the mean), credits are issued against the conservative lower bound rather than the mean. Computing the uncertainty percentage and the conservative estimate is a few lines:

half_width <- (mc_ci[2] - mc_ci[1]) / 2
unc_pct    <- 100 * half_width / mean(sim)
conservative <- mean(sim) - half_width           # lower-bound crediting basis
round(c(uncertainty_pct = unc_pct,
        mean = mean(sim), conservative = conservative), 1)
#> uncertainty_pct.97.5%                  mean    conservative.97.5% 
#>                  38.9                   3.3                   2.0

If the uncertainty percentage clears the threshold, the gap between the mean and the conservative figure is credit the project forgoes for being imprecise, the direct financial incentive to measure better.

## 7.6 Keystone: area-adjusted accuracy

The Bolivia land-cover work raises the same question for areas rather than carbon: a class area read straight off a map inherits the map’s classification errors, and good practice adjusts it and attaches a confidence interval (Olofsson et al., 2014). This is the LULC counterpart of the carbon deduction, and it uses the confusion matrix from the disturbance chapter. We take mapped forest and non-forest areas and an error matrix, and compute the area-adjusted estimate:

A    <- c(forest = 18000, nonforest = 7000)      # mapped area, ha
Atot <- sum(A); W <- A / Atot

# error matrix: rows = map class, cols = reference class
n   <- matrix(c(95, 5, 12, 88), nrow = 2, byrow = TRUE,
              dimnames = list(map = names(A), ref = names(A)))
ni  <- rowSums(n)

# Olofsson stratified estimator of the reference area proportion of "forest"
p_forest <- sum(W * n[, "forest"] / ni)
se_p     <- sqrt(sum(W^2 * (n[, "forest"]/ni) * (1 - n[, "forest"]/ni) / (ni - 1)))

adj_forest_ha <- Atot * p_forest
ci_ha         <- Atot * 1.96 * se_p
round(c(mapped_forest_ha  = A["forest"],
        adjusted_forest_ha = adj_forest_ha,
        ci_halfwidth_ha    = ci_ha), 0)
#> mapped_forest_ha.forest      adjusted_forest_ha         ci_halfwidth_ha 
#>                   18000                   17940                     893

The adjusted area differs from the mapped area because the map commits and omits forest at rates the reference sample reveals, and the confidence interval is the honest precision of the figure. A project that reports only the mapped area, with no adjustment and no interval, has reported a number no auditor can accept. The full Bolivia workflow applies this class by class across the national legend; the estimator is the one above.

## 7.7 Exercises

- Recompute the sampling confidence interval using a 4-by-6 quadrat grid instead of 8-by-12. Does the mean change? Does the standard error? Explain the difference.

- In the Monte Carlo, double the uncertainty on the carbon fraction (sd 0.04) and rerun. How much does the uncertainty percentage grow, and would it now trigger a 15% deduction?

- Report the conservative (lower-bound) carbon density and express the forgone credit as a percentage of the mean. In one sentence, state the incentive this creates.

- (Applied.) Change the error matrix so the map omits more forest (change the second row to 25, 75). Recompute the adjusted forest area and its interval, and explain which way omission error pushes the adjustment.

# 8. Time Series

## 8.1 Data cubes

Everything so far has been a snapshot. Monitoring is about change, and change lives in time, so the capstone data structure is the space-time cube: a stack of raster layers sharing one grid, indexed by date. A pixel is no longer a single value but a time series, and the questions become temporal, is this pixel trending down, when did it drop, how fast is it recovering.

The word cube is meant literally. The data have three axes and you must hold all three in mind at once. Two are spatial, x and y, the familiar grid of the earlier chapters. The third is time, t, and it is the one that changes how everything must be handled, because unlike x and y it is not sampled on a regular grid unless you force it to be. A satellite revisits on a fixed cycle but returns a usable observation only when the sky is clear, so the raw time axis is ragged. Operational cubes carry a fourth axis as well, the spectral band, which is why real archives are four-dimensional arrays and why the software built for them, gdalcubes and sits in R, is organised around regularising all four at once (Appel & Pebesma, 2019; Simões et al., 2021).

We build a monthly index cube over the plot spanning nine years. It carries three things a real optical record carries: a slow downward trend, an annual seasonal swing from leaf-on to leaf-off, and observation noise. A fire clears a block in mid-2021 and holds it low.

library(terra)
library(sf)

stems <- st_as_sf(read.csv("data/scbi_stems.csv"),
                  coords = c("lon", "lat"), crs = 4326) |>
  st_transform(32617)
r <- rast(ext(vect(stems)), resolution = 10, crs = "EPSG:32617")

dates <- seq(as.Date("2016-01-15"), as.Date("2024-12-15"), by = "month")
doy   <- as.numeric(format(dates, "%j"))
yfrac <- as.numeric(format(dates, "%Y")) + (doy - 1) / 365

set.seed(3)
xy   <- xyFromCell(r, 1:ncell(r))
burn <- xy[, 1] > 747700 & xy[, 1] < 747850 &
        xy[, 2] > 4309000 & xy[, 2] < 4309150

# simulated: trend + seasonal cycle + noise, with a 2021 fire in the burn box
mcube <- rast(lapply(seq_along(dates), function(k) {
  seas <- 0.25 * sin(2 * pi * (doy[k] - 100) / 365)
  v <- 0.62 - 0.008 * (yfrac[k] - 2016) + seas + rnorm(ncell(r), 0, 0.02)
  if (yfrac[k] >= 2021.5) {
    v[burn] <- 0.20 + 0.06 * sin(2 * pi * (doy[k] - 100) / 365) +
               rnorm(sum(burn), 0, 0.02)
  }
  setValues(r, v)
}))
names(mcube) <- format(dates, "%Y-%m")
terra::time(mcube) <- dates
c(layers = nlyr(mcube), cells = ncell(mcube))
#> layers  cells 
#>    108   2688

## 8.2 The two planes

A cube can only be looked at one slice at a time, and there are two ways to cut it. Fix a date and you get the spatial plane: a map, every pixel at one instant, the view every earlier chapter of this book has used. Fix a pixel and you get the temporal plane: a profile, one location across every date, the view that makes change visible. Neither is complete. A map cannot tell you whether a low value is a burn scar or simply February, and a profile cannot tell you whether a decline is one pixel or the whole stand.

Schematic of a space-time cube. Each horizontal sheet is one date. Cutting horizontally gives the spatial plane, a map; cutting vertically through a single pixel gives the temporal plane, a profile.

Cut the cube both ways on the same data. Three dates from 2019, an undisturbed year, show the spatial plane swinging with the season while the map itself barely changes:

plot(mcube[[c("2019-02", "2019-07", "2019-11")]],
     range = c(0.2, 0.95),
     main = c("February 2019", "July 2019", "November 2019"))

The same three maps differ by more than half an index unit, and nothing on the ground has changed. Now the temporal plane, two pixels drawn across all 108 dates, one inside the burn box and one outside it:

inside  <- cellFromXY(r, cbind(747775, 4309075))   # in the 2021 burn
outside <- cellFromXY(r, cbind(747600, 4309300))   # undisturbed

p_in  <- as.numeric(mcube[inside])
p_out <- as.numeric(mcube[outside])

plot(dates, p_out, type = "l", col = "#2f5d4e", lwd = 1.6,
     ylim = c(0.1, 1.0), xlab = "", ylab = "index",
     main = "Temporal plane: two pixels")
lines(dates, p_in, col = "#b5342e", lwd = 1.6)
legend("bottomleft", c("undisturbed", "burned 2021"),
       col = c("#2f5d4e", "#b5342e"), lwd = 1.6, bty = "n", cex = 0.85)

The saw-tooth is the seasonal cycle, present in both pixels throughout. The step down in the red line during 2021 is the fire. Note how much larger the annual swing is than the trend it rides on: the disturbance is obvious because it is abrupt, but the slow decline underneath is nearly invisible against the seasonality.

## 8.3 Seasonality and randomness

That last observation is the central problem of the temporal plane, and it is worth quantifying rather than asserting. A time series of this shape is conventionally treated as three additive parts: a trend, the slow directional movement; a seasonal component, the oscillation that repeats each year; and a remainder, what is left when both have been removed. Seasonal-trend decomposition by loess separates them without assuming the seasonal shape is a fixed sine (Cleveland et al., 1990). Base R implements it in stl(), which needs the profile declared as a regular series with twelve observations per cycle:

s   <- ts(p_out, start = c(2016, 1), frequency = 12)
dec <- stl(s, s.window = "periodic")
plot(dec, main = "Seasonal-trend decomposition, undisturbed pixel")

The three panels beneath the data are the decomposition. Partitioning the variance among them says which one governs what an analyst sees:

v <- apply(dec$time.series, 2, var)
round(100 * v / sum(v), 1)                 # per cent of variance
#>  seasonal     trend remainder 
#>      97.5       1.4       1.1
round(c(remainder_sd = sd(dec$time.series[, "remainder"])), 4)
#> remainder_sd 
#>       0.0191

Seasonality accounts for almost all of the variance in the record. The trend, the thing carbon monitoring exists to measure, is a percent or two. The remainder is smaller still, and its standard deviation recovers the observation noise that was put into the simulation, which is the check that the decomposition is doing what it claims.

This is the distinction the whole chapter turns on. The seasonal component is nature’s oscillation: deterministic, repeating, and therefore removable, either by decomposition or simply by always comparing like with like. The remainder is nature’s randomness together with sensor error: irreducible, and the honest floor on how precisely any single observation can be known. Confusing the two is expensive. Seasonality mistaken for randomness inflates the error bars you report and costs credits to the uncertainty deduction. Seasonality mistaken for trend invents change that never happened.

## 8.4 Sampling and aliasing

The practical consequence appears the moment you reduce a monthly record to an annual one, which every monitoring programme does. If each year is represented by an observation taken at the same point in the season, the seasonal component is a constant offset and cancels out of the comparison. If the dates drift, the seasonal component leaks into the difference between years and is indistinguishable from real change. Signal processing calls this aliasing. In forest monitoring it is the reason cloud cover biases change estimates, because the dates you can see are not a random sample of the dates you wanted (Hansen et al., 2013).

Estimate the trend both ways on the same pixel. First from a consistent July observation each year, then from one observation per year taken at whatever month happened to be clear:

yrs   <- 2016:2024
julys <- which(as.numeric(format(dates, "%m")) == 7)
fixed_slope <- unname(coef(lm(p_out[julys] ~ yrs))[2])

set.seed(11)
drift_slope <- replicate(2000, {
  idx <- sapply(yrs, function(y) sample(which(format(dates, "%Y") == as.character(y)), 1))
  coef(lm(p_out[idx] ~ yrs))[2]
})

round(c(true          = -0.008,
        fixed_july    = fixed_slope,
        drift_mean    = mean(drift_slope),
        drift_sd      = sd(drift_slope)), 5)
#>       true fixed_july drift_mean   drift_sd 
#>   -0.00800   -0.00755   -0.00720    0.02265
round(quantile(drift_slope, c(0.025, 0.975)), 4)
#>    2.5%   97.5% 
#> -0.0499  0.0365

Consistent sampling recovers the true rate of decline closely. Drifting sampling recovers it on average, because the seasonal error is centred rather than systematic, but the spread around that average is several times the size of the trend being measured. The interval spans zero comfortably in both directions:

round(100 * mean(drift_slope > 0), 1)      # per cent of draws with the wrong sign
#> [1] 37.9

More than a third of the time, an analyst sampling this way would report a pixel as greening when it is in fact declining. Nothing is wrong with the sensor, the algorithm, or the arithmetic. The error is entirely a consequence of sampling a periodic signal at inconsistent phase, and no amount of downstream statistics recovers what the sampling design threw away.

This is where the chapter connects back to the previous one. Chapter 7 propagated uncertainty through an equation, taking the input distributions as given. Here the input distribution is itself a product of a design decision. Compositing to a fixed seasonal window before any comparison is not data cleaning, it is uncertainty reduction, and it is the cheapest available: it costs nothing but discipline and it removes a variance component that dwarfs the quantity being estimated.

## 8.5 Compositing to an annual cube

So we reduce the monthly cube to one layer per year, taking a consistent mid-summer window rather than an annual mean, which would average leaf-on against leaf-off and discard the contrast that makes disturbance visible. This is the temporal normalisation step that operational cube software performs with cloud-ranked compositing:

years <- 2016:2024
cube <- rast(lapply(years, function(y) {
  sel <- which(format(dates, "%Y") == as.character(y) &
               as.numeric(format(dates, "%m")) %in% 6:8)
  app(mcube[[sel]], median)
}))
names(cube) <- paste0("y", years)
nlyr(cube)
#> [1] 9

## 8.6 Temporal reduction

A cube is reduced along time the way a single raster is summarised along space. app() applies a function to each pixel’s time series. The mean over years is the baseline surface; the standard deviation flags where the record is unstable:

mean_ndvi <- app(cube, mean)
sd_ndvi   <- app(cube, sd)
plot(c(mean_ndvi, sd_ndvi), main = c("Mean index", "Temporal SD"))

High temporal SD is the signature of disturbance: stable forest barely varies year to year once the season is controlled, so the burned block lights up.

## 8.7 Trends

The direction and rate of change per pixel is the slope of index against year, a linear trend fitted independently at every location. app() carries the regression across the cube:

slope <- app(cube, function(v) {
  if (all(is.na(v))) return(NA)
  coef(lm(v ~ years))[2]
})
names(slope) <- "trend_per_year"
plot(slope, main = "Per-pixel trend (index / year)")

Strong negative slopes mark sustained loss. This is the engine of trend-based monitoring: a whole landscape of time series reduced to one map of who is gaining and who is losing.

## 8.8 Detecting the event

A trend says a pixel declined; monitoring needs to know when. Differencing consecutive years isolates the step. The fire year is the layer where the largest drop occurs, and thresholding the year-on-year change delineates the burned area:

d2021 <- cube[["y2021"]] - cube[["y2020"]]
burned <- d2021 < -0.3
cell_ha  <- prod(res(r)) / 1e4
burned_ha <- global(burned, "sum", na.rm = TRUE)[1, 1] * cell_ha
round(burned_ha, 2)                                # hectares burned in 2021
#> [1] 2.25

## 8.9 Keystone: IPCC Tier 1 fire emissions

The capstone joins the whole book: a time series detects a fire, the burned area is measured, and IPCC Tier 1 methods turn that area into an emission. The IPCC-wildfire-emissions workflow runs this chain from burn-area mapping through to emission factors, and the governing equation is the same one every fire inventory uses (IPCC, 2006):

Lfire=A×MB×Cf×Gef×10−3

where A is burned area, MB the fuel biomass available, Cf the combustion factor, and Gef the emission factor for the gas. With the burned area from the cube and IPCC default factors for temperate forest:

A   <- burned_ha        # ha, from the time-series detection
MB  <- 120              # tonnes dry matter per ha (fuel load)
Cf  <- 0.45             # combustion factor (fraction consumed)
Gef <- 1569             # g CO2 per kg dry matter (IPCC default)

CO2_tonnes <- A * MB * Cf * Gef * 1e-3
round(c(burned_ha = A, CO2_tonnes = CO2_tonnes), 1)
#>  burned_ha CO2_tonnes 
#>        2.3      190.8

That single number, tonnes of CO2 from the fire, is the end of the pipeline this course has built toward: geospatial data, cleaned and projected in the early chapters, turned into a detected, measured, and quantified emission with every step reproducible. In a real inventory each factor carries its own uncertainty, so the honest final move is to wrap this equation in the Monte Carlo of the previous chapter and report the emission with a confidence interval. The methods are all now in hand.

set.seed(9)
sim <- replicate(2000, {
  mb  <- rnorm(1, 120, 20)
  cf  <- rnorm(1, 0.45, 0.08)
  gef <- rnorm(1, 1569, 100)
  A * mb * cf * gef * 1e-3
})
round(quantile(sim, c(0.025, 0.5, 0.975)), 0)      # CO2 tonnes: lower, median, upper
#>  2.5%   50% 97.5% 
#>   105   190   292

Note what that interval does and does not include. It propagates the uncertainty in the three emission factors while treating the burned area A as known exactly, which it is not: it came from a threshold applied to a differenced cube, and both the threshold and the dates feeding it are choices. The area-adjusted accuracy of Chapter 7 is how that term is estimated properly, and a complete Tier 2 statement would carry it into this same simulation alongside the factors.

## 8.10 Exercises

- Reduce the cube with app() to the maximum index per pixel instead of the mean. Where does the maximum surface differ most from the mean, and why there?

- Fit the per-pixel trend on the pre-fire years only (2016–2020). How does the burned block’s slope compare with the full-record slope, and what does that say about mixing a step change into a linear trend?

- Recompute burned area with a −0.2 threshold on the 2021 difference. How does the emission estimate move, and which term in the IPCC equation is it multiplying?

- Run stl() on the burned pixel p_in rather than the undisturbed one. Which component absorbs the 2021 step, and what does that tell you about applying seasonal decomposition across a disturbance?

- Rebuild the annual cube using an annual mean over all twelve months instead of a summer median. How do the temporal SD map and the detected burned area change, and why does averaging across the season blur the event?

- Repeat the drifting-date experiment restricting the sampled months to June through September, as a cloud-screened archive in a seasonal climate might allow. How much of the variance inflation survives, and what does that imply about how tight a compositing window needs to be?

- (Applied.) Using the Monte Carlo output, report the median CO2 and its 95% interval, then state which of the three factors (fuel load, combustion factor, emission factor) you would most want to measure locally to narrow the interval.

# 9. Spatial Patterns

## 9.1 Point patterns

Every earlier chapter analysed values measured at locations: carbon at a plot, elevation at a pixel, an index through time. Point-pattern analysis asks a different question, about the locations themselves. Are the events, tree stems, seedlings, detected clearings, spread evenly, scattered at random, or clustered together? The answer characterises the process that generated them, and it is the second capstone of this course: a whole branch of spatial statistics with its own tools. We use spatstat, whose ppp object holds a point pattern inside an observation window.

The running example here is a real regeneration survey: conifer seedlings mapped after the 2015 Mt Midgeley wildfire in the Darkwoods Conservation Area of the southern Selkirk Mountains, British Columbia (Murphy et al., 2026). We load the seedlings and build a ppp from one four-hectare plot, its bounding box serving as the window:

library(spatstat)

seed <- read.csv("data/darkwoods_seedlings.csv")
p3   <- subset(seed, plot == 3)
win  <- owin(range(p3$x), range(p3$y))
pp   <- ppp(p3$x, p3$y, window = win)
pp
#> Planar point pattern: 1607 points
#> window: rectangle = [524164.7, 524362.6] x [5446884, 5447083] units

The printout reports the number of points and the window: a little over 1,600 seedlings in a 200 by 200 metre plot. That is the pattern whose structure we now interrogate.

## 9.2 Spatial randomness

The reference against which every pattern is judged is complete spatial randomness (CSR): points falling independently and with equal probability everywhere, a homogeneous Poisson process. Real patterns depart from CSR in one of two directions, toward regularity, where points avoid each other as in a planted stand, or toward clustering, where points attract as in natural regeneration around surviving seed trees. The whole toolkit exists to measure that departure and test whether it is real. A plot of the seedlings already suggests the answer:

plot(pp, main = "Mapped conifer seedlings, plot 3", pch = 20, cex = 0.3, cols = "forestgreen")

## 9.3 Intensity

The intensity is the first-order property: the expected number of points per unit area, and how it varies across the window. A kernel density estimate smooths the points into a continuous intensity surface, the point-pattern analogue of a heat map, revealing where regeneration concentrated:

plot(density(pp, sigma = 8), main = "Seedling intensity (points/m²)")
points(pp, pch = 20, cex = 0.2)

Regeneration is patchy, not uniform: dense pockets sit near surviving seed sources and gentler, wetter ground, while other areas stay bare. That spatial unevenness is the signal, and explaining it is what the disturbance and terrain covariates in the study were for.

## 9.4 Nearest neighbour

Whether points cluster or repel is the second-order property, about the interaction between them. The most direct diagnostic is the nearest-neighbour distance: how far, on average, each seedling sits from its closest neighbour, compared with what CSR predicts. Under CSR the expected mean nearest-neighbour distance is 1/2λ, so the ratio of observed to expected is the Clark–Evans index: below one means clustered, above one means regular.

ce_ratio <- function(df) {
  q <- ppp(df$x, df$y, window = win)
  obs <- mean(nndist(q))
  expected <- 1 / (2 * sqrt(intensity(q)))
  obs / expected
}
round(ce_ratio(p3), 3)          # < 1 indicates clustering
#> [1] 0.821

The ratio sits well below one: seedlings are closer to one another than randomness would allow, the quantitative signature of clustered regeneration.

## 9.5 Ripley’s K

A single number cannot capture how clustering changes with scale, points may cluster tightly at short range yet disperse at long range. Ripley’s K function describes the pattern across all distances at once, counting on average how many other points lie within a growing radius of each point, against the CSR expectation. Plotting the observed K with a simulation envelope is the standard test:

K <- envelope(pp, Kest, nsim = 19, verbose = FALSE)
plot(K, main = "Ripley's K with CSR envelope")

Where the observed curve rises above the envelope the pattern is more clustered than random at that distance; below it, more regular. The seedlings sit above the envelope across the mapped range, confirming clustering is real and not an artefact of a few tight groups, and the scale-explicit view is why K, and its variance-stabilised form the L function, is the workhorse of the field.

## 9.6 Keystone: seedlings

The published study runs exactly this arc, then goes one step further (Murphy et al., 2026). Across 3,886 mapped seedlings on three four-hectare plots, point-pattern analysis found significant clustering for all seven conifer species, with intensity governed by fire severity, distance to seed source, topography, and wind. The lean diagnostics above reproduce its core finding on one plot: clustered regeneration, quantified by the Clark–Evans ratio and confirmed by Ripley’s K. Repeating the Clark–Evans ratio species by species shows the clustering is not confined to the dominant lodgepole pine but holds across the community:

spp <- c("Pinus_contorta", "Larix", "Picea", "Pseudotsuga")
sapply(spp, function(s) {
  q <- subset(p3, species == s)
  if (nrow(q) < 20) return(NA)
  round(ce_ratio(q), 3)
})
#> Pinus_contorta          Larix          Picea    Pseudotsuga 
#>          0.859          0.881          0.973          0.934

Every species returns a ratio below one. The paper then models the intensity surface against the disturbance and terrain covariates with inhomogeneous Cox process models, showing that areas of overlapping beetle mortality and high-severity fire had significantly reduced pine regeneration. That covariate model is the natural next step beyond this book: the foundations here, CSR, intensity, nearest-neighbour, Ripley’s K, are what it is built on, and an auditor who understands them can read and challenge such a result.

## 9.7 Exercises

- Build the ppp for plot 1 instead of plot 3 and compute its Clark–Evans ratio. Is the regeneration there clustered, random, or regular, and how does it compare with plot 3?

- Map the intensity surface with a larger smoothing bandwidth (sigma = 20) and a smaller one (sigma = 4). How does the choice change what you see, and what does that imply about reporting a single heat map?

- Run envelope(pp, Kest) on a random thinning of the seedlings (rthin(pp, 0.2)). Does the clustering signal survive when only a fifth of the points remain? What does that say about sample size and power?

- (Applied.) Detected clearings in a monitoring image are also a point pattern. Explain how Ripley’s K could distinguish scattered natural tree-fall from an organised, clustered harvest front, and why that distinction matters for attributing disturbance.

# Appendix A — Automation, Protocols and Registries

## A.1 Beyond the analysis

A defensible result is only half the job. The other half is the discipline around it: making the work reproducible, governing the data to a recognised standard, and delivering credits into a registry that will accept them. This appendix sketches that surrounding practice. Its code is illustrative and is not run at render time.

## A.2 Reproducible runtimes

The analysis in this book runs today; the test is whether it runs in three years, on another machine, for a reviewer who was not there. Two habits carry most of that weight. Pin your package versions so an update cannot silently change a result, and script the whole pipeline end to end so there are no undocumented manual steps.

# Pin the project's package library to exact versions
renv::init()        # snapshot the library
renv::snapshot()    # write renv.lock, committed with the project
renv::restore()     # a reviewer rebuilds the exact environment

A single entry point that runs every stage in order is the other half. When the raw data change, one command regenerates every figure and number, and nothing depends on remembering which script to run first.

# run.R — the one command that rebuilds everything
source("01-ingest.R")       # read and validate raw survey data
source("02-process.R")      # projection, extraction, modelling
source("03-report.R")       # render outputs with quarto::quarto_render()

## A.3 Data protocols and QA/QC

Every number inherits the quality of the survey that produced it. Standardised digital capture, enforced schemas, and a logged accuracy estimate on every GPS fix are what let an auditor trust a plot table at all. The governing reference is the geospatial data-quality framework of the ISO 19100 series, which defines the quality elements, completeness, logical consistency, positional accuracy, thematic accuracy, an auditor should expect to see documented. In practice this means a QA/QC step that checks geometry validity, coordinate ranges, attribute domains, and duplicate records before any analysis begins.

# Minimal ingest QA gate
stopifnot(
  all(sf::st_is_valid(layer)),                     # geometry validity
  all(sf::st_coordinates(layer)[, 1] > -180),      # coordinates in range
  !anyDuplicated(layer$plot_id)                    # no duplicate keys
)

## A.4 Registries

Credits live or die at the registry. ART-TREES, Verra (VCS), and ACR each publish methodologies, uncertainty and accuracy requirements, and validation and verification procedures that a project must meet before issuance. The analytical chapters of this book map directly onto their requirements: the area check onto boundary and area rules, the accuracy assessment onto the mandatory confusion matrix, the Monte Carlo onto the uncertainty deduction, the fire-emissions chain onto the reporting equations. Knowing which registry a project targets, and reading its methodology as the specification the analysis must satisfy, is the final piece of the auditor’s craft.

## A.5 Further practice

The workflows referenced here, standard operating procedures, demonstration repositories with renv and version control, safeguards and QA/QC templates, and data-service automation, are where the habits of this book become an operational system. The analysis is the visible part; the reproducibility, governance, and registry alignment are what make it hold up under scrutiny.

# Appendix B — References

Appel, M., & Pebesma, E. (2019). On-demand processing of data cubes from satellite image collections with the gdalcubes library. Data, 4(3), 92. https://doi.org/10.3390/data4030092

Bivand, R. S., Pebesma, E., & Gómez-Rubio, V. (2013). Applied spatial data analysis with r (2nd ed.). Springer.

Cleveland, R. B., Cleveland, W. S., McRae, J. E., & Terpenning, I. (1990). STL: A seasonal-trend decomposition procedure based on loess. Journal of Official Statistics, 6(1), 3–73.

Hansen, M. C., Potapov, P. V., Moore, R., Hancher, M., Turubanova, S. A., Tyukavina, A., Thau, D., Stehman, S. V., Goetz, S. J., Loveland, T. R., Kommareddy, A., Egorov, A., Chini, L., Justice, C. O., & Townshend, J. R. G. (2013). High-resolution global maps of 21st-century forest cover change. Science, 342(6160), 850–853. https://doi.org/10.1126/science.1244693

Hijmans, R. J. (2024). Terra: Spatial data analysis. https://CRAN.R-project.org/package=terra

IPCC. (2006). 2006 IPCC guidelines for national greenhouse gas inventories, volume 4: Agriculture, forestry and other land use. Institute for Global Environmental Strategies.

Jenkins, J. C., Chojnacky, D. C., Heath, L. S., & Birdsey, R. A. (2003). National-scale biomass estimators for united states tree species. Forest Science, 49(1), 12–35. https://doi.org/10.1093/forestscience/49.1.12

Lovelace, R., Nowosad, J., & Muenchow, J. (2019). Geocomputation with r. Chapman; Hall/CRC.

Murphy, S., Leslie, A., Wilson, J., & Banks, L. K. (2026). Spatial patterns of conifer regeneration following mountain pine beetle outbreak (dendroctonus ponderosae) and mixed-severity wildfire: A point process model of disturbance interactions in the southern selkirk mountains, british columbia. Forest Ecology and Management, 618, 123985. https://doi.org/10.1016/j.foreco.2026.123985

Olofsson, P., Foody, G. M., Herold, M., Stehman, S. V., Woodcock, C. E., & Wulder, M. A. (2014). Good practices for estimating area and assessing accuracy of land change. Remote Sensing of Environment, 148, 42–57. https://doi.org/10.1016/j.rse.2014.02.015

Pebesma, E. (2018). Simple features for r: Standardized support for spatial vector data. The R Journal, 10(1), 439–446. https://doi.org/10.32614/RJ-2018-009

Pebesma, E., & Bivand, R. (2023). Spatial data science: With applications in r. Chapman; Hall/CRC. https://doi.org/10.1201/9780429459016

Simões, R., Camara, G., Queiroz, G., Souza, F., Andrade, P. R., Santos, L., Carvalho, A., & Ferreira, K. (2021). Satellite image time series analysis for big earth observation data. Remote Sensing, 13(13), 2428. https://doi.org/10.3390/rs13132428